Processing

Clock cycle assumptions:

ADC sampling frequency (per ADC) is 1 MHz.

2 ADC channels for Excitation, interleaved between samples.

2 ADC channels for Sense/Response, interleaved between samples.

Emulated ADC sampling by interleaving (2 for Excitation + 2 for Sense) ⇒ effective 2 MHz simultaneous Excitation and Sense

Cycle/MAC calculation:

Buffer length = 2000 samples (Excitation and Sense each). Filling one full buffer takes 1 ms ⇒ impedance throughput 1 kHz.

Ping-pong approach: while first halves are filling, MAC the previously filled second halves with reference sin and cos.

For one frequency, per 1 ms: 4000 MAC (Exc) + 4000 MAC (Resp) ⇒ 8000 MAC/ms/freq.

Fs=2MHz⇒2000 samples/ms.

Per freq, per sample: 4 MACs (Exc I/Q + Resp I/Q) ⇒ 8000 MAC/ms/freq.

Core: 200 MHz ⇒ 200 000 cycles/ms.

2 MAC = 1 cycle (dual 16×16), ignore overhead.

Nmax=200000×28000=50 (ideal case, no overhead).

50 frequencies in ideal case, without overhead, using only 1 core.

Memory

On-chip memory:

Flash: 512 KB (256 KW) or 1 MB (512 KW), ECC-protected

RAM: 172 KB (86 KW) or 204 KB (102 KW), ECC- or parity-protected

Reference signals are pre-calculated and stored in memory (RAM or Flash).

Four configurations are considered:

Worst case: RAM, low-end model → 172 KB (86 KW)

Next worst: RAM, high-end model → 204 KB (102 KW)

Better case: Flash, low-end model → 512 KB (256 KW)

Best case: Flash, high-end model → 1 MB (512 KW)

Note: All calculations are in 16-bit words (W).

ADC data buffers:

Excitation = 2000 W, Response = 2000 W → total 4000 W.

These are dynamic and must reside only in RAM, independent of the number of frequencies (fixed overhead).

Reference data per frequency:

Each frequency requires 2000 W for sine + 2000 W for cosine →

4000 W/frequency + 4000 W(ADC).

Separate buffers are used for sine and cosine (no quarter-period optimization) to simplify indexing.

| Case | Memory Type | Total Capacity (Words) | Reference Location | Available for References | Max Number of Frequencies |
| --- | --- | --- | --- | --- | --- |
| 1 | RAM – Low-end (172 KB) | 86 000 | RAM | 82 000 | 20 |
| 2 | RAM – High-end (204 KB) | 102 000 | RAM | 98 000 | 24 |
| 3 | Flash – Low-end (512 KB) | 256 000 | Flash | 256 000 | 64 |
| 4 | Flash – High-end (1 MB) | 512 000 | Flash | 512 000 | 128 |